I want to Use MPU in STM32H743 with FreeRTOS. I have already ported the FreeRTOS for this controller with the Dynamic allocation support and basic features (like Delay, Stack Hooks, Idle Hook, etc..) enabled. I have also used the hardware Timer2 for the system Tick handling. Please provide me step by step process from now how to enable the MPU and how to use it. Also explain the MPU features and how it is used with detailed explanations. Please communicate in English language only.

# Enabling MPU with FreeRTOS on STM32H743: A Complete Step-by-Step Guide

Enabling the Memory Protection Unit (MPU) with FreeRTOS on your STM32H743 microcontroller will significantly enhance the robustness and security of your embedded application. The MPU acts as a hardware-enforced boundary, isolating tasks from each other and preventing a fault in one task from corrupting the entire system.

Since you already have FreeRTOS ported with dynamic allocation support, stack hooks, idle hook, and Timer2 configured for system tick, you are well-prepared for this next step. Here is a comprehensive, step-by-step guide to enable and use the MPU.

## 📌 Step 1: Understanding Key Concepts Before You Start

Before diving into the implementation, it is crucial to understand the fundamental concepts behind FreeRTOS MPU support.

### Task Privilege Levels

When the MPU is enabled, tasks can run in one of two modes:

| Mode | Description | Capabilities |
| --- | --- | --- |
| Privileged Mode | Full access to the entire memory map and all peripherals. | Can execute any instruction and access any memory address. Used for critical system tasks and the kernel itself. |
| Unprivileged (User) Mode | Restricted access. Can only access memory regions explicitly assigned to it. | Cannot access memory outside its defined regions. Attempting to do so triggers a MemManage Fault. Used for application tasks to improve isolation. |

### Memory Regions

For an unprivileged task to function, you must define memory regions that specify exactly which parts of memory the task is allowed to access. Each region is defined using the MemoryRegion_t structure, which includes:

- Base Address: The starting address of the region.

- Length: The size of the region in bytes (must be a power of two).

- Parameters: Access permissions (read, write, execute, etc.).

### Key MPU APIs

You will use specialized FreeRTOS APIs to create tasks with MPU protection:

- xTaskCreateRestricted(): Creates a task with MPU protection using dynamically allocated memory.

- xTaskCreateRestrictedStatic(): Creates a task with MPU protection using statically allocated memory.

- vTaskAllocateMPURegions(): Modifies the MPU regions of an existing task at runtime.

Important: You cannot use the standard xTaskCreate() to create tasks that need MPU protection. You must use the restricted versions.

## ⚙️ Step 2: Modify FreeRTOSConfig.h

This is the most critical configuration step. You need to add or modify the following macros in your FreeRTOSConfig.h file.

### Enable MPU Support

c

#define configENABLE_MPU 1

This macro enables the FreeRTOS MPU functionality. Without this, the MPU-specific APIs and port layer will not be activated.

### Configure the Number of MPU Regions

c

#define configTOTAL_MPU_REGIONS 16

The STM32H743's MPU hardware supports up to 16 programmable memory regions. You must inform the FreeRTOS kernel of this number so it can manage them correctly.

### Ensure the Correct Port Layer

You must ensure your project is using the correct FreeRTOS portable layer that supports MPU. The appropriate file is located at:

text

FreeRTOS/Source/portable/GCC/ARM_CM4_MPU/port.c

Although the folder name says ARM_CM4, this port layer is fully compatible with the Cortex-M7 core in your STM32H743 and supports all 16 MPU regions.

### Include Required Headers

In your application source files, ensure you include:

c

#include "task.h"

This header contains the MPU-specific API declarations.

## 🛠️ Step 3: Create MPU-Protected Tasks

Now it is time to create your first MPU-protected task. The process involves defining memory regions, packaging them into a task definition structure, and then creating the task.

### 3.1 Define the Task Function

First, write your task function as you normally would. For this example, we will create a simple LED flashing task.

c

void vFlashTask(void *pvParameters)

{

/* Task implementation */

while(1)

{

/* Toggle LED */

vTaskDelay(pdMS_TO_TICKS(500));

}

}

### 3.2 Define the Task Stack

You need to allocate a stack for your task. For MPU-protected tasks, you must use statically allocated stacks (even when using the dynamic task creation API).

c

/* Define the task stack with proper alignment */

static portSTACK_TYPE vFlashTaskStack[ configMINIMAL_STACK_SIZE ] __attribute__((aligned(8)));

Note: The stack must be 8-byte aligned for Cortex-M7 compatibility.

### 3.3 Define Memory Regions

Now, define the memory regions that your task will be allowed to access. The three typical regions are:

- Code Region: The task's executable code (usually read-only).

- Data Region: Any global or static data the task needs to access (read-write).

- Stack Region: The task's stack memory (read-write).

c

static const MemoryRegion_t xFlashRegions[] =

{

/* Region 0: Task code (read-only, executable) */

{

.pvBaseAddress = (void *) &vFlashTask,

.ulLengthInBytes = 256, /* Adjust to actual code size */

.ulParameters = portMPU_REGION_READ_ONLY | portMPU_REGION_EXECUTE_NEVER

},

/* Region 1: Task data (read-write) */

{

.pvBaseAddress = (void *) 0x20000000, /* Example data region */

.ulLengthInBytes = 1024,

.ulParameters = portMPU_REGION_READ_WRITE

},

/* Region 2: Task stack (read-write) */

{

.pvBaseAddress = (void *) vFlashTaskStack,

.ulLengthInBytes = sizeof(vFlashTaskStack),

.ulParameters = portMPU_REGION_READ_WRITE

}

};

Critical Constraints:

- Each region's ulLengthInBytes must be a power of two (e.g., 32, 64, 128, 256, 512, 1024, etc.).

- The region's base address (pvBaseAddress) must be aligned to the same power-of-two boundary as its length.

- The stack region's length must be large enough to hold the entire stack.

### 3.4 Define the Task Parameters Structure

Combine the task function, stack, priority, and memory regions into a TaskParameters_t structure.

c

TaskParameters_t xFlashTaskDef =

{

.pvTaskCode = vFlashTask,

.pcName = "Flash Task",

.usStackDepth = sizeof(vFlashTaskStack) / sizeof(portSTACK_TYPE),

.pvParameters = NULL,

.uxPriority = tskIDLE_PRIORITY, /* Runs in Unprivileged mode */

.puxStackBuffer = vFlashTaskStack,

.xRegions = xFlashRegions

};

Understanding Task Priority and Privilege:

- If uxPriority is set to a standard priority (e.g., tskIDLE_PRIORITY), the task runs in Unprivileged (User) mode.

- If uxPriority includes the portPRIVILEGE_BIT (e.g., tskIDLE_PRIORITY | portPRIVILEGE_BIT), the task runs in Privileged mode.

For most application tasks, you will want them to run in Unprivileged mode for maximum isolation and protection.

### 3.5 Create the Task

Finally, create the task in your main() function.

c

TaskHandle_t xFlashTaskHandle = NULL;

int main(void)

{

/* Hardware initialization */

HAL_Init();

SystemClock_Config();

/* ... other initialization ... */

/* Create MPU-protected task */

if (xTaskCreateRestricted(&xFlashTaskDef, &xFlashTaskHandle) != pdPASS)

{

/* Task creation failed - handle error */

while(1);

}

/* Start the scheduler */

vTaskStartScheduler();

/* Should never reach here */

while(1);

}

## ⚠️ STM32H7 Special Considerations

The STM32H7 series has some unique characteristics that require special attention when using the MPU.

### Cache Coherency Issue

The STM32H7 has a powerful cache system (I-Cache and D-Cache). If the FreeRTOS heap is placed in AXI SRAM and D-Cache is enabled, you may encounter cache coherency issues that can lead to unpredictable behavior or hard faults.

Solution: Configure the MPU region covering the FreeRTOS heap as Non-Cacheable.

c

/* Define a region for the FreeRTOS heap (adjust addresses as needed) */

extern uint8_t __heap_start;

extern uint8_t __heap_end;

static const MemoryRegion_t xHeapRegion[] =

{

{

.pvBaseAddress = &__heap_start,

.ulLengthInBytes = (uint32_t)&__heap_end - (uint32_t)&__heap_start,

.ulParameters = portMPU_REGION_READ_WRITE | portMPU_REGION_NON_CACHEABLE

}

};

Alternatively, you can place the FreeRTOS heap in DTCM RAM, which is not cached by default and avoids this issue entirely.

### Ensure Proper Alignment

All MPU regions must be aligned to their size. For example, a 1KB region must start at an address that is a multiple of 1024. This is particularly important for the stack region, which must be aligned to the stack's size.

## 📝 Step 4: Additional Configuration and Best Practices

### Define a Fault Handler

When a task attempts to access memory outside its permitted regions, a MemManage Fault will occur. You should implement a fault handler to capture and log these errors for debugging.

c

void MemManage_Handler(void)

{

/* Log the fault and task information */

/* You can call vTaskGetInfo() to get details about the offending task */

/* Optionally reset the system or take corrective action */

while(1);

}

### Create Privileged Tasks When Necessary

Some tasks, such as device drivers or system management tasks, may need full system access. For these, you can create privileged tasks by setting their priority with the portPRIVILEGE_BIT.

c

TaskParameters_t xPrivilegedTaskDef =

{

/* ... other fields ... */

.uxPriority = tskIDLE_PRIORITY | portPRIVILEGE_BIT,

/* ... */

};

### Use vTaskAllocateMPURegions() for Dynamic Region Changes

If you need to change a task's memory regions at runtime (e.g., to grant temporary access to a peripheral), use the vTaskAllocateMPURegions() API.

c

MemoryRegion_t xNewRegions[3] = { /* New region definitions */ };

vTaskAllocateMPURegions(xTaskHandle, xNewRegions);

## 📚 Step 5: Testing and Debugging

### Verify Your MPU Configuration

After implementing MPU protection, thoroughly test your system:

- Intentional Violation Test: Temporarily modify your task code to access memory outside its defined regions. Verify that a MemManage Fault is triggered, confirming that the MPU is working correctly.

- Stack Overflow Test: Reduce the stack size for a task and force a stack overflow. The MPU should detect the overflow and trigger a fault.

- Performance Impact: The MPU adds minimal overhead (typically less than 1% of CPU time). However, you should measure the impact on your specific application if performance is critical.

### Use Debugging Tools

- JTAG/SWD Debugger: Use your debugger to inspect the MPU configuration registers and verify that the regions are set up correctly.

- FreeRTOS Trace Tools: Tools like FreeRTOS+Trace can provide insight into task behavior and help identify violations.

## 🎯 Summary

You have successfully enabled MPU functionality with FreeRTOS on your STM32H743. The key steps were:

- Understood concepts: Privileged vs. Unprivileged modes, memory regions, and MPU-specific APIs.

- Modified FreeRTOSConfig.h: Enabled MPU support and configured the number of regions.

- Created MPU-protected tasks: Defined memory regions and used xTaskCreateRestricted().

- Handled STM32H7 specifics: Managed cache coherency issues and ensured proper alignment.

- Implemented best practices: Added fault handlers and tested the system.

## 📖 Recommended Resources

To deepen your understanding, I highly recommend the following resources:

| Resource | Description |
| --- | --- |
| FreeRTOS MPU Support Page | Official documentation with detailed explanations and examples. |
| FreeRTOS MPU Demo Project | The official demo project for STM32H743ZI. This is the best learning resource. |
| STM32 MPU Tips - ST MOOC | STMicroelectronics online training course on using the MPU with STM32F7/H7. |
| FreeRTOS API Reference - xTaskCreateRestricted | Official API documentation for creating MPU-protected tasks. |

## ❓ Frequently Asked Questions

### Q: Can I use dynamic memory allocation with MPU-protected tasks?

A: Yes, but the task's stack must still be statically allocated. The xTaskCreateRestricted() API expects a pre-allocated stack buffer. The task's control block (TCB) and other kernel objects can still be allocated dynamically from the FreeRTOS heap.

### Q: What happens if an unprivileged task calls an API function that requires privileged access?

A: FreeRTOS handles this automatically. The kernel temporarily elevates the task's privilege level to execute the API call, then restores the original privilege level. This means you can call any FreeRTOS API from an unprivileged task without manual intervention.

### Q: How many MPU regions can I use per task?

A: The configTOTAL_MPU_REGIONS macro defines the maximum number of regions per task. For STM32H743, this is typically set to 16. However, you should use only as many regions as necessary, as each region consumes hardware MPU resources.

### Q: Can I create both privileged and unprivileged tasks in the same application?

A: Absolutely. You can mix and match privileged and unprivileged tasks as needed. The kernel itself runs in privileged mode, while application tasks can be either privileged or unprivileged based on your design requirements.

Can I use the xTaskCreate() function and create the normal task in MPU supported FreeRTOS just like I do in Non-MPU supported versions?

## Can I Use xTaskCreate() with MPU-Enabled FreeRTOS?

Yes, you can, but it is strongly discouraged and poses significant security risks.

In an MPU-enabled FreeRTOS system, the xTaskCreate() function remains available. However, the "normal" tasks it creates behave fundamentally differently than they do in non-MPU versions, and this can lead to system instability or even crashes.

### ⚠️ The Behavior and Risks of xTaskCreate() with MPU Enabled

- Task Privilege Depends on the Priority Parameter:

  - xTaskCreate() can create a task that runs either in Privileged Mode or User Mode.

  - This is controlled by the uxPriority parameter:

    - If you set the portPRIVILEGE_BIT flag (e.g., uxPriority = 2 | portPRIVILEGE_BIT), the task will have unrestricted access to the entire memory map.

    - If you do not set this flag, the task runs in User Mode.

- User Mode Tasks Are Dangerously Insecure:

  - According to the official FreeRTOS documentation, xTaskCreate() should only be used to create tasks that require unrestricted access to the entire microcontroller memory map.

  - This means that even if a task is created in User Mode via xTaskCreate(), its MPU regions may not be correctly configured. In practice, this means the task might still be able to access memory it is not supposed to, completely defeating the purpose of the MPU and exposing your system to severe memory corruption and security vulnerabilities.

### ✅ The Correct Approach: Use xTaskCreateRestricted()

To fully benefit from the MPU's hardware-enforced memory protection, the official documentation strongly recommends using xTaskCreateRestricted() instead of xTaskCreate().

- xTaskCreateRestricted() is the dedicated API designed specifically for FreeRTOS-MPU.

- It allows you to explicitly define and assign protected MPU memory regions for a task at the time of creation, ensuring that the task can only access the resources it is explicitly authorized to use.

### 💎 Summary

| API | Recommended for MPU? | What It Does |
| --- | --- | --- |
| xTaskCreate() | No | Creates a task with no MPU protection; its access is not properly restricted, defeating the purpose of the MPU. |
| xTaskCreateRestricted() | Yes | Creates a task with full MPU protection; you define precise memory regions it can access. |

### 📌 Conclusion

While xTaskCreate() is technically available, using it in an MPU-enabled system is a bad practice that compromises the very security and robustness you are trying to achieve. For all MPU-protected tasks in your application, you should always use xTaskCreateRestricted() (or its static version xTaskCreateRestrictedStatic()).

I have noted your preference for English. All future communications will be in English only.

What is the "configSYSTEM_CALL_STACK_SIZE" and what should be the value in STM32H743ZI?

## What is configSYSTEM_CALL_STACK_SIZE and What Should Be Its Value for STM32H743ZI?

configSYSTEM_CALL_STACK_SIZE is a configuration macro introduced in FreeRTOS V10.6.0. It is primarily used in scenarios where the MPU (Memory Protection Unit) is enabled.

### 💡 What Does This Macro Do?

Before this feature was introduced, FreeRTOS system calls (such as API functions called from a task) used the stack of the task that made the call. This posed a security risk when the MPU was enabled.

The new MPU enhancement solves this by allocating a separate, privileged-mode-only stack within the Task Control Block (TCB). The configSYSTEM_CALL_STACK_SIZE macro controls the size of this independent system call stack. The benefits are:

- Enhanced Security: Critical kernel operations are isolated from user task stacks.

- Stack Overflow Protection: It prevents complex user task stack overflows from corrupting the system call execution environment.

### 🎯 Choosing the Right Value for STM32H743ZI

There is no fixed standard for this value, as it depends entirely on the complexity and nesting depth of system calls in your specific application.

- Important Unit: The value of this macro is specified in words. On a 32-bit architecture like the STM32H743ZI (Cortex-M7), 1 word = 4 bytes.

- Recommended Starting Value: For the STM32H743ZI, a reasonable and safe starting value is between 128 and 256 words (which translates to 512 to 1024 bytes).

  - 128 words (512 bytes): This is a common baseline value and is sufficient for simple system calls.

  - 256 words (1024 bytes): Choose this if your tasks call more complex FreeRTOS APIs or if there is significant interrupt nesting. It provides a larger safety margin.

- Be Cautious – Don't Set It Too High: It is critical to understand that this stack is part of every task's TCB. This means that whatever size you set for configSYSTEM_CALL_STACK_SIZE will be multiplied by the number of tasks you create, potentially consuming a large amount of RAM.

  - For example, setting configSYSTEM_CALL_STACK_SIZE to 2048 words (8192 bytes) would make every task's TCB enormous, and you could quickly exhaust your available RAM.

### 💎 Practical Recommendations for Your Project

- Start with a Conservative Value: Define it in your FreeRTOSConfig.h as follows:

c

#define configSYSTEM_CALL_STACK_SIZE    ( 128 )

- Perform Thorough Stress Testing: Run your application and exercise all operations that trigger system calls.

- Monitor and Adjust: If your system crashes or experiences HardFault during complex operations, try increasing the value to 192 or 256. If everything runs smoothly and you want to conserve RAM, you can try reducing it further.

### 📌 Summary

For the STM32H743ZI, starting with 128 words (512 bytes) is an excellent choice. Always test and adjust this value based on your actual application's behavior, and be mindful not to set it too high to avoid wasting valuable RAM resources.

Note: All future responses will be provided in English as per your preference.